SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across  networks. (BL2, BL3)

Assessment Tool Weightage: 30%

Annexure A

CONCEPT MAPPING

Code of Conduct:

I __R.Vishnu(192572180)__________________ (Name / Reg No) certify that this submission is my original work and  that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the student:

Annexure A

CONCEPT MAPPING

1. A partially completed concept map is given below:

Application → Presentation → ______ → Transport → Network → ______ → Physical.

Analyze the missing layers and explain how their functions are essential for reliable  communication.

Missing OSI Layers

Concept Map:
Application → Presentation → Session → Transport → Network → Data Link → Physical

Analysis:
The missing layers are Session Layer and Data Link Layer. The Session Layer is responsible for establishing, managing, maintaining, and terminating communication sessions between two devices. It also provides synchronization by creating checkpoints so that communication can resume from the last saved point if a failure occurs. The Data Link Layer provides reliable node-to-node communication by converting packets into frames, adding physical addresses (MAC addresses), detecting transmission errors using techniques such as CRC, and controlling access to the transmission medium. Together, these layers improve reliability, reduce communication errors, and ensure that data is delivered correctly between sender and receiver.

2. Construct a concept map linking data units (Segment, Packet, Frame, Bits) with their  respective OSI layers and analyze how transformation of data units supports  transmission.

Data Units and Their OSI Layers

Concept Map:

- Transport Layer → Segment

- Network Layer → Packet

- Data Link Layer → Frame

- Physical Layer → Bits

Analysis:
As data travels through the OSI model, each layer changes the format of the data to perform specific communication tasks. At the Transport Layer, application data is divided into segments for reliable delivery. The Network Layer adds logical addressing information, converting each segment into a packet. The Data Link Layer adds MAC addresses and error detection information, converting packets into frames. Finally, the Physical Layer converts frames into bits, which are transmitted as electrical, optical, or wireless signals. At the receiving end, the process is reversed, allowing the original data to be reconstructed accurately. This transformation enables efficient, organized, and reliable communication across networks.

3.Given the concept map:

DataLink→MACAddress→LocalDelivery

Network → IP Address → Global Delivery

Explain how these relationships ensure complete data delivery across networks.

MAC Address and IP Address Relationship

Concept Map:

- Data Link Layer → MAC Address → Local Delivery

- Network Layer → IP Address → Global Delivery

Analysis:
The Data Link Layer uses MAC (Media Access Control) addresses to identify devices within the same local network. MAC addresses are unique hardware addresses permanently assigned to network interface cards and are used for local communication. The Network Layer uses IP addresses to identify devices across different networks and determine the best route for packets to reach their destinations. During transmission, routers use IP addresses to forward packets between networks, while switches use MAC addresses to deliver frames within a local network. The combination of MAC and IP addressing ensures complete end-to-end communication by enabling both local and global data delivery.

4. A concept map shows:

ErrorControl

→DataLinkLayer→ErrorDetection

→ Transport Layer → Error Correction

Analyze how the interaction between these layers improves reliability in data  transmission.

Error Control in Data Link and Transport Layers

Concept Map:

- Data Link Layer → Error Detection

- Transport Layer → Error Correction

Analysis:
The Data Link Layer ensures that frames transmitted between directly connected devices are free from errors by using techniques such as CRC (Cyclic Redundancy Check), parity bits, and checksums. If an error is detected, the frame is discarded or retransmission may be requested depending on the protocol. The Transport Layer, particularly TCP, provides end-to-end reliability by detecting lost or damaged segments, sending acknowledgments (ACKs), and retransmitting missing data. It also maintains the correct sequence of segments before delivering them to the application. The interaction between these two layers greatly improves communication reliability by minimizing data loss and ensuring accurate data delivery.

5. Develop a concept map for web browsing using OSI layers and analyze how failure  in one layer affects the entire communication process.

Concept Map:

Application (Web Browser)
↓
Presentation (SSL/TLS Encryption, Data Formatting)
↓
Session (Creates and Maintains Communication Session)
↓
Transport (TCP Segmentation, Flow Control, Error Recovery)
↓
Network (IP Addressing and Routing)
↓
Data Link (Framing, MAC Address, Error Detection)
↓
Physical (Transmission of Bits)

Analysis:
Web browsing requires all seven OSI layers to function together. The Application Layer provides services through a web browser such as Chrome or Firefox. The Presentation Layer encrypts data using SSL/TLS and converts data into a suitable format. The Session Layer establishes and manages the communication session between the client and web server. The Transport Layer uses TCP to provide reliable communication through segmentation, acknowledgments, and flow control. The Network Layer routes packets using IP addresses. The Data Link Layer delivers frames within the local network and detects transmission errors. Finally, the Physical Layer transmits bits through cables, fiber optics, or wireless media. If any layer fails, the entire communication process is affected. For example, if the Transport Layer fails, packets may be lost, preventing the web page from loading even if the other layers are functioning correctly.

6. A network issue is represented in the concept map:

Physical ✔ → Data Link ✔ → Network ✖ → Transport ✖ → Application ✖ Analyze the problem and explain how the concept map helps in troubleshooting.

Troubleshooting Using the OSI Concept Map

Concept Map:

Physical ✔ → Data Link ✔ → Network ✖ → Transport ✖ → Application ✖

Analysis:
The concept map indicates that the Physical Layer and Data Link Layer are operating correctly. This means that the hardware components such as cables, network interface cards, switches, and local frame transmission are functioning properly. However, the Network Layer has failed, which could be due to incorrect IP addressing, routing problems, router failure, or network configuration issues. Since the Network Layer cannot deliver packets to the destination, the Transport Layer cannot establish reliable end-to-end communication, and the Application Layer cannot provide services such as web browsing, email, or file transfer. This layered troubleshooting approach allows network administrators to identify the faulty layer quickly, reducing diagnosis time and simplifying network maintenance. The concept map provides a systematic method for locating communication problems by checking each OSI layer one at a time.

Top of Form

Bottom of Form

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Concept Identification | 20% | Accurately identifies all relevant OSI layers, concepts, and relationships | Identifies most concepts with minor omissions | Identifies limited concepts; some inaccuracies | Unable to identify key concepts correctly |
| Concept Mapping Structure | 25% | Constructs a clear, logical, and well organized concept map with proper hierarchy and connections | Concept map is mostly clear with minor structural issues | Concept map lacks clarity, organization, or proper flow | No clear structure or incorrect mapping |
| Linking & Relationships | 20% | Clearly establishes correct relationships between layers, functions, and processes | Relationships are mostly correct with minor errors | Limited or partially correct relationships | Incorrect or missing relationships |
| Application & Analysis | 20% | Effectively applies concepts to explain processes (e.g., encapsulation, error control) with strong reasoning | The application is correct but lacks depth in analysis | Basic application with weak explanation | Unable to apply concepts correctly |
| Explanation & Clarity | 15% | Provides a clear, concise, and well structured explanation supporting the concept map | Explanation is understandable with minor gaps | Explanation is unclear or incomplete | No clear or meaningful explanation |